Image based Features

- Mean: The average pixel intensity value within the image. It gives a sense of the overall brightness of the image.

- Sigma (Standard Deviation): A measure of the spread or dispersion of pixel intensity values from the mean. Higher sigma indicates greater variation in pixel intensities.

- Skewness: A measure of the asymmetry of the distribution of pixel intensities.

- Kurtosis: A measure of the "tailedness" of the distribution of pixel intensities; it indicates the presence of outliers.

- Contrast: Measures the local variations in pixel intensities. Higher contrast indicates larger differences between adjacent pixel values.

- Energy: Represents the uniformity or smoothness of the image texture. High energy values indicate more texture variation.

- Correlation: A measure of how correlated a pixel is to its neighbors over the entire image.

- Homogeneity: Reflects the closeness of pixel intensity values in the image. High homogeneity indicates that neighboring pixel intensities are similar.

- Dissimilarity: Measures the average absolute difference in pixel intensity values between neighboring pixels.

- ASM (Angular Second Moment): Also known as Energy, it is a measure of texture uniformity and homogeneity, calculated as the sum of squares of pixel intensities in the Gray Level Co-occurrence Matrix (GLCM).

- Max Probability: The maximum probability of occurrence of a certain texture pattern in the image.

- LBP (Local Binary Pattern) features: Descriptors that capture the local texture patterns by comparing each pixel with its neighboring pixels and encoding the result as a binary number.

- LTP (Local Ternary Pattern) features:  Descriptors that but capture local ternary patterns.

- HOG (Histogram of Oriented Gradients) features: Descriptors that capture the distribution of gradient orientations in different parts of the image.